#TN2078	Rev. A   	[http://www.rainbow.com/tech/notes.html]

Distribution:	End User

TITLE: 	Sentinel System Driver May Not Auto Detect PCI-based Parallel Ports

Date: November 3, 2000

Summary: 	Workaround to get the Sentinel System Driver to recognize a PCI-based parallel port.

Distribution:	The distribution category for this technical note is “End User.” It may be distributed to all Rainbow Engineering, Sales, Technical Support, Distributors, Developers, and Rainbow end users.

The Sentinel System driver install process will not recognize PCI-based parallel ports with non-standard address locations. We provide a workaround that allows the Sentinel System Driver to recognize the PCI-based parallel port. These instructions assume that the system drivers are already installed and can be used to communicate with LPT1 (usually address 378 hex):

- Get the resource properties for the PCI parallel port through the Windows Control Panel. (The actual icon in the Control Panel you use to display port properties depends on the version of Windows you are using.) In some cases, a range rather than an absolute address is displayed (e.g., D400-D407).

- Launch Sentw9x.exe (for Windows 9x) or Setupx86.exe (for Windows NT or Windows 2000) and choose Functions>Configure>Add from the drop-down menu.

- Under bus address (phy addr is not available using sentw9x.exe), enter the address or the lower end of the range (D400) and make sure that the radio button for Use this port is set to NO. Failing to do this can result in an endless loop of messages about “install the board drivers,” “port conflict”...”install board drivers,” “port conflict” and so on.

- Reboot the computer and launch Sentw9x.exe or Setupx86.exe again. Choose Functions>Configure>Edit from the drop-down menu and switch the Use this Port radio button to Yes. Complete the editing and exit the setup program.

- If on Windows 9x or Windows ME, it is recommended that you reboot at the completion of step 4.